Annex 6. Collection and Transportation

Note: based on the Preinvestment Technical Design Study (in Spanish: Estudio de Diseño Técnico de Preinversión (EDTP)) the following information is presented as a reference.

Having defined the particularities of the collection service to be implemented in the analysis of alternatives, including the utilization of recyclable waste and the processing for compost production, the quantities of solid waste allocated to each component of the collection service to be designed are specified.

- Service Design

In this case, the generation of recyclable waste is considered along with the results of the market study regarding the types of marketable waste in the city of Sucre and other cities in the country. Consequently, for the purposes of this project, the following fractions are not considered: other types of paper, polystyrene, other plastics, and flat glass, due to the lack of potential buyers in the Sucre market. This group of waste is categorized as Discarded Recyclable Solid Waste (DRSW). Based on this consideration, the following table has been generated:

| Year | Total Recyclable Waste Generated (Tons/Year) | Total, Generated without DRSW | Recovery Rate (%) | Annual Recovered Quantity (Tons/Year) | Final Production Percentage (%) | Approximate Final Production (Tons/Year) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 119,67 | 114,06 | 15% | 17,11 | 60% | 10,27 |
| 1 | 120,41 | 114,76 | 15% | 17,21 | 60% | 10,33 |
| 2 | 121,17 | 115,49 | 15% | 17,32 | 60% | 10,39 |
| 3 | 121,91 | 116,19 | 15% | 17,43 | 60% | 10,46 |
| 4 | 122,66 | 116,90 | 15% | 17,54 | 60% | 10,52 |
| 5 | 123,44 | 117,64 | 15% | 17,65 | 60% | 10,59 |
| 6 | 124,19 | 118,36 | 30% | 35,51 | 70% | 24,86 |
| 7 | 124,95 | 119,08 | 30% | 35,72 | 70% | 25,01 |
| 8 | 125,74 | 119,84 | 30% | 35,95 | 70% | 25,17 |
| 9 | 126,51 | 120,57 | 30% | 36,17 | 70% | 25,32 |
| 10 | 127,28 | 121,31 | 30% | 36,39 | 70% | 25,47 |
| 11 | 128,09 | 122,08 | 45% | 54,94 | 80% | 43,95 |
| 12 | 128,87 | 122,83 | 45% | 55,27 | 80% | 44,22 |
| 13 | 129,67 | 123,58 | 45% | 55,61 | 80% | 44,49 |
| 14 | 130,49 | 124,36 | 45% | 55,96 | 80% | 44,77 |
| 15 | 131,32 | 125,16 | 45% | 56,32 | 80% | 45,06 |
| 16 | 132,13 | 125,93 | 60% | 75,56 | 90% | 68,00 |
| 17 | 132,97 | 126,73 | 60% | 76,04 | 90% | 68,43 |
| 18 | 133,79 | 127,51 | 60% | 76,50 | 90% | 68,85 |
| 19 | 134,61 | 128,29 | 60% | 76,97 | 90% | 69,28 |
| 20 | 135,46 | 129,10 | 60% | 77,46 | 90% | 69,72 |

Similarly, the quantities allocated for compost production have been determined as follows:

| Year | Total Organic Waste (Tons/Year) | Annual Recovery Rate (%) | Annual Recovered Quantity (Tons/Year) | Final Production Percentage (%) | Approximate Final Production (Compost) (Tons/Year) |
| --- | --- | --- | --- | --- | --- |
| 0 | 205.36 | 15% | 30.80 | 20% | 6.16 |
| 1 | 206.61 | 15% | 30.99 | 20% | 6.20 |
| 2 | 207.91 | 15% | 31.19 | 20% | 6.24 |
| 3 | 209.18 | 15% | 31.38 | 20% | 6.28 |
| 4 | 210.46 | 15% | 31.57 | 20% | 6.31 |
| 5 | 211.79 | 15% | 31.77 | 20% | 6.35 |
| 6 | 213.08 | 30% | 63.93 | 20% | 12.79 |
| 7 | 214.38 | 30% | 64.31 | 20% | 12.86 |
| 8 | 215.75 | 30% | 64.73 | 20% | 12.95 |
| 9 | 217.07 | 30% | 65.12 | 20% | 13.02 |
| 10 | 218.39 | 30% | 65.52 | 20% | 13.10 |
| 11 | 219.78 | 45% | 98.90 | 20% | 19.78 |
| 12 | 221.12 | 45% | 99.51 | 20% | 19.90 |
| 13 | 222.48 | 45% | 100.12 | 20% | 20.02 |
| 14 | 223.89 | 45% | 100.75 | 20% | 20.15 |
| 15 | 225.33 | 45% | 101.40 | 20% | 20.28 |
| 16 | 226.71 | 60% | 136.03 | 20% | 27.21 |
| 17 | 228.15 | 60% | 136.89 | 20% | 27.38 |
| 18 | 229.55 | 60% | 137.73 | 20% | 27.55 |
| 19 | 230.96 | 60% | 138.57 | 20% | 27.71 |
| 20 | 232.43 | 60% | 139.46 | 20% | 27.89 |

The selected alternative considers implementing differentiated collection and transportation in 100% of the intervention area. Considering the population size, existing activities, and the amount of waste generated, the following differentiated waste collection has been established:

- Organic solid waste,

- Recyclable solid waste,

- Non-recoverable solid waste,

- Battery waste,

- Bio infectious solid waste.

Type of Users

The users include:

- Urban households

- Rural households

- Restaurants

- Lodging facilities

- Markets

- Public and private institutions

- Educational units

- Alcantarí Airport

User Universe and Zonal Distribution

These user groups are organized into the following zones:

| Users | Area | Neighborhoods |
| --- | --- | --- |
| Urban households | Urban | 7 neighborhoods, with 470 families in year 0 and 482 families in year 20. |
| Rural households | Rural | Populated centers of 7 communities, with 869 families in year 0 and 891 families in year 20. |
| Restaurants | Urban | 5 units located in the Central Axis and Bolívar neighborhoods. |
| Lodging facilities | Urban | 1 unit located in the Central Axis neighborhood. |
| Market | Urban | 1 unit located in the Bolívar neighborhood. |
| Public institutions | Urban | 3 units: GAMY*, District Education Office, and Police, all located in the Central Axis neighborhood. |
| Private institutions | Urban | 1 banking entity located in the Bolívar neighborhood. |
| Urban schools | Urban | 2 units located in the Central Axis neighborhood. |
| Rural schools | Rural | 3 units located in the communities of Escana, Pulqui Avaroa, and Molle Punku. |
| Alcantarí Airport | Rural | 1 unit located in the Alcantarí community. |

*GAMY: Gobierno Autónomo Municipal de Yamparáez

Coverage to be Achieved

The goal is to achieve 90% coverage. That is, from the current coverage of 40%, an increase to 90% is expected.

- Service Route Mapping

For this case, three sectors have been determined for organizing service provision, each divided into areas that will later allow for the organization of micro-routes (Annex a).

The previous graphic presents the sectorization of the collection and transportation service, which has been organized as follows:

- Sector 1: Composed of the urban area and Alcantarí Airport.

- Sector 2: Composed of the populated centers of the communities of Paccha Pata, Quirahuani, Alcantarí, and Pulqui Avaroa.

- Sector 3: Composed of the populated centers of the communities of Molle Punku, Lavadero, and Escana.

This sectorization considers the location of urban neighborhoods and rural communities. Please annex b to the micro routes for the solid waste collection.

The estimated amount of waste generated by sector is detailed in the following table:

| Sector | Areas per Sector | Waste Generated (Tons/Day) |
| --- | --- | --- |
| 1 | Urban area and Alcantarí Airport | 0.44 |
| 2 | Populated centers of Paccha Pata, Quirahuani, Alcantarí, and Pulqui Avaroa | 0.37 |
| 3 | Populated centers of Molle Punku, Lavadero, and Escana | 0.39 |
| Total |  | 1.20 |

Macro-route Design

Subsequently in this process we subdivided into areas to then elaborate the routes and sections. Specifically, for sector 1, it has been organized in 4 areas, according to the following imagine (Annex c):

As observed, there are four work areas that respond to an analysis of roads and connections, detailed as follows:

- Area 1: Comprising the neighborhoods of Eje Central, Bolívar, and Morro.

- Area 2: Comprising the neighborhoods of San José, 14 de Abril, and América.

- Area 3: Comprising the neighborhood of San Isidro.

- Area 4: Comprising District 3, where an educational unit is currently located.

For the populated centers of rural community areas, only a macro-route is assigned, as shown in the following images.

The following corresponds to the macro-routes from the rural area sector 2 (Annex d).

These areas correspond to:

- Area 1: Populated center of the community of Paccha Pata.

- Area 2: Populated center of the community of Quirahuani.

- Area 3: Populated center of the community of Alcantarí.

- Area 4: Populated center of the community of Pulqui Avaroa.

Next, the image for distribution of the macro-routes for sector 3 (Annex e).

- Area 1: Populated center of the community of Escana.

- Area 2: Populated center of the community of Lavadero.

- Area 3: Populated center of the community of Molle Punku.

Micro-route Design

The following criteria have been considered for the design of micro-routes:

- The route starts at the Garage and ends at the final disposal site.

- It minimizes U-turns and left turns.

- It is continuous, without dead zones or overlapping areas.

- In closed streets, collection is carried out in reverse motion and forward motion for collection.

- In hillside areas, collection starts at the higher elevations and moves downward.

- Along the route, the volume of waste collected from large generators is considered.

A heuristic method has been applied for the design of the micro-routes. The respective diagrams are presented below (Annex f).

As observed in the diagram, the established routes ensure service coverage for all households. A field verification of accessibility has been conducted, defining the routes along roads accessible to the designated collection vehicles.

The route starts from the GAMY Garage and ends in the San Isidro neighborhood. The following table presents the distances traveled for each area and the estimated times for service provision.

| Area | Route | Start | End | Distance (Km) | Coverage |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | GAMY Garage | Barrio Morro | 4.59 | Eje Central, Bolívar, Morro |
| 2 | 2 | Barrio 14 de Abril | Barrio San José | 3.1 | 14 de Abril, América, San José |
| 4 | 4 | Barrio San José | Airport | 21.97 | U.E. Eduardo Sempertegui, Alcantarí Airport |
| 3 | 3 | Airport | San Isidro | 1.62 | San Isidro |

| Area | Route | Departure Time | Speed (km/hr) | Travel Time (hrs) | Stop Time | End Time |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8:00 AM | 10 | 0.46 | 0.10 | 8:56 AM |
| 2 | 2 | 8:56 AM | 10 | 0.31 | 0.10 | 9:37 AM |
| 4 | 4 | 9:37 AM | 50 | 0.44 | 0.20 | 10:11 AM |
| 3 | 3 | 10:11 AM | 10 | 0.16 | 0.40 | 11:04 AM |

The following diagrams and details are also presented for micro-routes in Sector 2 (Annex g).

| Area | Route | Start | End | Distance (Km) | Coverage |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | GAMY Garage | Paccha Pata Community | 7.99 | Paccha Pata Community |
| 2 | 2 | Paccha Pata Community | Quirahuani Community | 5.57 | Quirahuani Community |
| 3 | 3 | Quirahuani Community | Alcantarí Community | 12.56 | Alcantarí Community |
| 4 | 4 | Alcantarí Community | Pulqui Avaroa Community | 13.58 | Pulqui Avaroa |

The following diagrams and details are also presented for micro-routes in Sector 3 (Annex h).

| Area | Route | Area | Beginning | End | Longitude (Km) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Garage GAMY | Community Escana | 20 | Community Escana |
| 2 | 2 | Community Escana | Community Lavadero | 13.65 | Community Lavadero |
| 3 | 3 | Community Lavadero | Community Molle Punku - San Isidro Salida | 18.81 | Community Molle Punku |

| Area | Route | Time of departure | Speed km/Hr | Time traveled Hrs | Time stops | Time end | Length traveled in community (Km) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8:00 a. m. | 30 | 0.67 | 0.20 | 9:27 a. m. | 3.97 |
| 2 | 2 | 9:27 a. m. | 30 | 0.46 | 0.12 | 10:35 a. m. | 2.35 |
| 3 | 3 | 10:35 a. m. | 30 | 0.63 | 0.40 | 11:38 a. m. | 2.26 |

The respective micro-routes for each community are shown in the annexes i to o.

Method of collection and transportation

The waste collection and transport method are categorized by sector and work area:

| Sector | Area | Method |
| --- | --- | --- |
| 1 | 1, 2, 3 | Curbside |
|  | 4 | Curbside and Corner Collection |
| 2 | 1 | Corner Collection |
|  | 2, 3, 4 | Curbside |
| 3 | 1 | Curbside |
|  | 2, 3 | Curbside and Corner Collection |

Collection Frequency and Schedule

In terms of collection frequency and schedules, the following is as follows:

| Frequency | Type of service | Service areas |
| --- | --- | --- |
| Daily Daytime | Waste collection from markets, businesses, institutions, homes | Central and outskirts of the urban area Rural area Airport |

The collection service starts at 08:00 AM and ends at 12:00 PM, operating five days a week.

Quantity and Sizing of Collection Equipment

Based on the amount of waste generated, the following is determined:

- 1 open-box truck of the dump truck type with a capacity of 6m³, divided into 2 to 3 compartments, in addition to added devices for the collection of recyclables, for the differentiated collection of 3 fractions: organic, recyclable, and non-recyclable.

- Added devices for the differentiated collection of bio-infectious waste.

- Added devices for the differentiated collection of batteries.

Collection Frequency by Macro Route and Collection Schedules

The frequency and collection schedules are as follows:

| Frequency | Type of Service | Areas of Attention |
| --- | --- | --- |
| Inter daily Daytime | Collection of waste from markets, businesses, institutions and homes | Central zone and periphery of the urban area, rural area, airport |
| Daily Daytime | Sweeping and cleaning waste collection | Roads and public areas |

The collection service starts at 08:00 AM and ends at 12:00 PM, operating five days a week.

Personnel Requirement, Drivers, and Assistants

The distribution of personnel for the quantity and type of vehicles indicated is as follows:

| Vehicle Type | Personnel |
| --- | --- |
| Open-box truck with division | 1 Driver, 2 Assistants |

For the established quantity of personnel, the following clothing and personal protective equipment, as well as required tools and annual supply frequency, are defined:

| Detail | Unit | Frequency of annual provisioning |
| --- | --- | --- |
| Overall with Reflectors | Piece | 2 |
| Cold Weather Parka | Piece | 1 |
| Long Sleeve Leather Gloves | Piece | 6 |
| Anti-fog Safety Glasses | Piece | 2 |
| Khaki Fabric Mouth Cover | Piece | 3 |
| Leather Boots | Piece | 1 |
| Hat or Cap | Piece | 1 |
| Triple Layer Masks | Box | 6 |
| Rain Poncho | Piece | 2 |
| Water Boots | Piece | 2 |
| Lumbar Belt | Piece | 1 |
| Shovel | Piece | 1 |
| Pitchfork | Piece | 1 |

Service Programming

The organization of the service is presented below:

| Sector | Description | Frequency | Monday | Tuesday | Wednesday | Thursday | Friday | Saturday |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sector 1 | Urban area and airport | 2 times a week | ✔ |  |  | ✔ |  |  |
| Sector 2 | Rural: Pacchapata, Quirahuani, Alcantarí, Pulqui, Avaroa | 2 times a week |  | ✔ |  |  | ✔ |  |
| Sector 3 | Rural: Molle Punku, Lavadero, Escana | 2 times a week |  |  | ✔ |  |  | ✔ |

Each day, the collection of the 3 fractions: organic, recyclable, and the rest is planned.

Differentiated Collection of Waste from Health Establishments

The service is expected to be performed 2 times a week, collecting waste generated in the establishments of:

- Centro de Salud Integral Cardenal Maurer

- Puesto de Salud de Escana

- Puesto de Salud de Lavadero

The waste will be collected on the same route as household and similar waste, but in a differentiated manner using the device. Therefore, the following quantity of waste is expected to be collected:

| Health facility | Amount of bio-infectious waste (Class A and B) Kg/day | Route |
| --- | --- | --- |
| Centro de Salud Integral Cardenal Maurer (Yamparáez) | 1,708 | 1 |
| Puesto de Salud Escana | 0.402 | 3 |
| Puesto de Salud Lavadero | 0.239 | 3 |

A device with a volume of 0.25 m³ is expected, with dimensions: 0.5x1x0.5m.

For batteries, green points are expected to be located at the following sites:

- Government Autonomous Municipal Facilities of Yamparáez

- Existing Bank Facilities in the Urban Area

- Educational Units in General, both Urban and Rural Areas

- Social Headquarters in the Rural Area

- Alcantarí International Airport Facilities

In this case, collection will be once a month, and a total of 74.54 kg/month is expected to be collected. A device of 0.30 m³ is expected, with dimensions: 0.5x1x0.60m.

Infrastructure and Civil Works Maintenance Area

The GAMY has a garage and maintenance area for the collection and transport vehicle.

- Storage

Primary Storage

According to Article 43 of the General Regulation of Law 755, for differentiated waste collection, systems for separation at the source and storage must be implemented. Waste collection systems must include the design of differentiated collection routes and other necessary mechanisms to maximize waste utilization.

According to the diagnosis carried out for the project, in its family survey, it is found that 90% of the population stores the solid waste generated within their home in plastic bags. About 30% reuse organic waste. And, 58% indicate a willingness to perform source separation of usable fractions.

Under Law 755, for household, non-household sources, and the airport, source separation into 3 fractions is established: organic, recyclable, and non-recyclable. Considering this contribution model, the population is expected to perform source separation of:

- Organic Fraction: Represented by the color green, can be delivered in buckets without a lid or with sufficient ventilation to avoid odors, should not contain impurities or contaminants, i.e., without bags.

- Recyclable Fraction: Identified by the color yellow, can be delivered in jute bags, boxes, or compact, orderly, and clean bundles. Cans, plastics, and all that can be should be crushed to reduce their volume. The following wastes enter this fraction:

| Recyclable Fraction | Type of materials within the fraction |
| --- | --- |
| Polyethylene terephthalate plastic (PET) | Among them: soft drink bottles, water bottles, cooking oil bottles. |
| High Density Polyethylene Plastics (HDPE) | Among them: milk containers, yogurt, cleaning products, liquid detergent containers, shampoo containers, buckets, pails. |
| Low density polyethylene plastic (LDPE) | Among them, dry cleaning bags, milk bags or similar, jutes, flexible lids. |
| Paper | Among them: white paper, notebooks, binder sheets, colored sheets. Must not be wet or greasy. |
| Cardboard, newspaper and similar | Among them: newspaper, magazines, books, brochures, cards, flyers, envelopes, cardboard boxes, cardboard tubes, folders, telephone directories, egg cartons. |
| Metallic containers | Among them: soft drink and canned food cans, cans of different types of beverages. Glass Of different types and colors. |

- Non-recyclable or Rest Fraction: Identified by the color black, can be delivered in well-tied bags, buckets, or jute bags.

Secondary Storage

Secondary storage will apply to large generators, such as schools, airports, festive activities that take place during the year, and their use in squares and public roads.

The number of containers is given under the following calculation:

- V: Volume of the storage container, in m³

- G: Generation of solid waste in kg/day

- P.V.: "In situ" volumetric weight of the waste, in kg/m³

- f: Collection frequency

- Fs: Safety factor

A safety factor of 1.25 will be adopted, considering that the current service will improve from regular to optimal.

Consequently, the volume value and the number of containers for different generators are:

Educational units

| Area | Educational Units | Quantity |
| --- | --- | --- |
| Urban | 2 | 1 Green of 180 liters, 1 Yellow of 270 liters, 1 Black of 270 liters |
| Rural | 5 | 5 units, 1 Green, Yellow, and Black of 50 liters. |

Airport

In the case of the airport, it is also necessary to assign containers for the external management of solid waste, with the following characteristics:

- 1 Green Container of 1100 Liters for organic waste

- 2 Yellow Containers of 1100 Liters for recyclable waste

- 2 Black Containers of 1100 Liters for non-recyclable waste.

However, to ensure that solid waste is sorted at source, the airport must guarantee the provision of differentiated containers in the interior corridors, offices, and food court.

Festive Activities

According to the Social Diagnosis carried out, a total of 2,295 people are expected to move in a year due to patron saint festivities, and therefore the provision of containers is foreseen, according to the following detail:

- 4 Green Containers of 1100 Liters for organic waste,

- 2 Yellow Containers of 1100 Liters for recyclable waste,

- 2 Black Containers of 1100 Liters for non-recyclable waste.

Olympic Mesh Containers for Recyclable Material – PET Bottles

To maximize recycling potential, avoid contamination of the Yamparáez River, and enhance environmental education among the visiting population, the implementation of Olympic mesh containers in different spaces of the project's intervention area is planned.

| Area | Location | Quantity | Location |
| --- | --- | --- | --- |
| Urban | Central area | 2 | Entrance and central area of Yamparáez |
| Rural | Community Paccha Pata | 1 | Main square |
| Rural | Community Quirahuani | 1 | Main square |
| Rural | Community Alcantarí | 1 | Main square |
| Rural | Community Pulqui Avaroa | 1 | Social Headquarters |
| Rural | Community Escana | 1 | Central part of the community |
| Rural | Community Lavadero | 1 | Central part of the community |
| Rural | Community Molle Punku | 1 | Social Headquarters and Main Square area |
| Rural | Airport | 1 | Entrance to the airport |
| TOTAL |  | 10 |  |

The technical specifications of the containers are detailed below:

| Capacity (L) | 50 | 180 | 270 | 1100 | 1700 |
| --- | --- | --- | --- | --- | --- |
| Height (mm) | 700 | 1016 | 1360 | 1395 | 1700 |
| Diameter (mm) | 372 | 605 | 606 | -- | -- |
| Length |  |  |  | 1350 | 1000 |
| Width |  |  |  | 1230 | 1000 |
| Other | Plastic without wheels, without metal support for educational units | Plastic without wheels – Arturito type | Plastic without wheels – Arturito type | With wheels with brake, Metal base, Plastic | Olympic mesh |
| Quantity | 15 units (5 green, 5 black, 5 yellow) | 2 units green | 4 units (2 yellow and 2 black) | 13 units (5 green, 4 yellow, 4 black) | 10 units |

A summary table of the number of containers, as defined, is presented below:

| Description | Quantity (piece) |
| --- | --- |
| Large Container with Olympic Mesh | 10 |
| Provision of Differentiated Containers of 1100 Liters | 13 |
| Provision of Differentiated Containers of 270 Liters | 4 |
| Provision of Containers of 180 Liters | 2 |
| Provision of Containers of 50 Liters | 15 |
| Provision of Closed Containers for Batteries of 10 Liters | 10 |